# GIS-BASED RENEWABLE ENERGY DECISION SUPPORT SYSTEM (DSS)

## 1. Conceptual Framework

The system implements a spatially explicit Decision Support System (DSS) for renewable energy planning, integrating raster-based multi-criteria suitability modelling with vector administrative boundaries and spatial database analytics. The core objective is to translate heterogeneous geospatial datasets into actionable planning indicators at both site and regional scales.

The methodology follows established GIS decision-analysis principles:

- Raster-based spatial modelling for continuous environmental variables

- Vector-based spatial aggregation for administrative reporting

- Zonal statistics and spatial joins for scale translation

- Spatial normalization to enable cross-regional comparability

## 2. Spatial Data Model

### 2.1 Raster Datasets (Continuous Spatial Variables)

The system operates on a set of georeferenced raster layers representing environmental and terrain constraints:

- Global Horizontal Irradiance (GHI)

- Wind Speed at reference height

- Wind Power Density (WPD)

- Terrain slope (derived from DEM)

- Composite solar suitability raster

- Composite wind suitability raster

Each raster represents a spatially continuous field, where pixel values encode either:

- physical resource intensity (e.g., GHI, WPD), or

- normalized suitability indices derived through multi-criteria evaluation.

All raster layers are stored in their native coordinate reference systems and dynamically reprojected during point-based sampling to preserve spatial accuracy.

### 2.2 Vector Datasets (Discrete Spatial Units)

Administrative boundaries (federal states) are represented as MULTIPOLYGON geometries stored in a PostGIS-enabled spatial database. These geometries serve as:

- Spatial masks for zonal aggregation

- Administrative units for policy-level reporting

- Containers for linking spatial statistics to governance structures

## 3. Suitability Modelling Approach

### 3.1 Multi-Criteria Evaluation (MCE)

Solar and wind suitability layers are not raw measurements but composite indices produced through raster-based MCE. The underlying workflow follows:

- Reclassification of raw environmental rasters into suitability classes

- Normalization of criteria to a common scale

- Weighted overlay to reflect relative importance of factors

- Generation of final suitability surfaces

This approach ensures:

- Dimensional consistency across heterogeneous datasets

- Explicit encoding of planning assumptions

- Spatial transparency in decision criteria

### 3.2 Suitability Classification

Final suitability rasters are discretized into ordinal classes:

- Lowest

- Low

- Moderate

- High

- Highest

These classes enable:

- Area-based suitability estimation

- Threshold-based land availability analysis

- Cartographically meaningful visualization

## 4. Scale-Dependent Analysis

### 4.1 Site-Level Analysis (Pixel Sampling)

At the site scale, the system performs direct raster cell sampling at user-defined geographic coordinates. This process:

- Reprojects input coordinates to raster CRS where necessary

- Extracts cell-level values from each raster layer

- Returns location-specific environmental and suitability metrics

This mode supports micrositing analysis, where local variability in resource availability and terrain constraints is critical.

### 4.2 Regional-Level Analysis (Zonal Statistics)

At the regional scale, raster values are aggregated within administrative boundaries using zonal statistical operations. Metrics such as mean values and area thresholds are precomputed to ensure computational efficiency and consistency.

This enables:

- Comparison of renewable energy potential across states

- Translation of raster variability into governance-aligned indicators

- Strategic, policy-oriented decision support

## 5. Interpretation of Reported Metrics

### 5.1 Highly Suitable Area (km²)

Highly suitable area represents the cumulative surface area of raster cells classified as high or very high suitability within a given administrative unit. Computation involves:

- Thresholding suitability rasters

- Counting qualifying pixels within state polygons

- Converting pixel counts to square kilometers

This metric quantifies spatial feasibility, not theoretical capacity, and is directly relevant to land-use planning.

### 5.2 Suitability Indices

Solar and wind suitability scores are spatially averaged indices derived from composite suitability rasters. These values:

- Represent mean spatial favorability

- Are normalized to facilitate inter-regional comparison

- Abstract spatial heterogeneity into interpretable planning indicators

They do not replace spatial detail but summarize it for decision-making contexts.

### 5.3 Environmental Characteristics

Mean GHI, wind speed, and WPD values are derived via zonal statistics applied to their respective rasters. These values characterize regional resource intensity, providing physical context for suitability outcomes.

Slope, derived from DEM analysis, functions as a terrain constraint. Its exclusion or null representation at regional scale reflects the high spatial variability of terrain and its greater relevance at site-specific resolution.

## 6. Spatialized Techno-Economic Estimation

Levelized Cost of Energy (LCOE) estimates are spatially differentiated by linking:

- Resource intensity

- Suitability indices

- Regional averages

GIS enables the transformation of LCOE from a non-spatial, national-average metric into a spatially variable indicator, reflecting geographic differences in resource quality. This integration exemplifies the role of GIS in coupling physical geography with economic modelling.

## 7. Decision Support Significance

From a GIS perspective, the system:

- Integrates raster and vector paradigms coherently

- Implements spatial multi-criteria decision analysis

- Supports multi-scale planning workflows

- Translates spatial complexity into policy-relevant indicators

The DSS demonstrates how geospatial analysis can operationalize renewable energy planning beyond visualization, functioning as a quantitative spatial reasoning framework.